Exact startup procedure — keep this for your documentation

Your current architecture is:

ESP32-S3

│

│ Wi-Fi / HTTP

▼

Phone Hotspot

│

├──────── Laptop

│             │

│        Flask RDT Server

│             │

│        Gradient Boosting

│             │

└─────────────┘

│

▼

ESP32 receives RDT

│

▼

OLED displays RDT

1. Before the viva — configure your phone hotspot

Turn on your phone hotspot and choose a stable SSID/password, for example:

SSID: Navod_RDT

Password: ********

Then connect your laptop to that phone hotspot.

Connect the ESP32 to the laptop by USB as usual for power/Serial Monitor.

The ESP32 also needs to join Navod_RDT over Wi-Fi.

2. Find the laptop's IP address

After connecting the laptop to the phone hotspot, open PowerShell:

ipconfig

Under the active Wi-Fi adapter, find:

IPv4 Address . . . . . . . . : 192.168.x.x

For example:

192.168.137.25

That address is important.

3. Put that IP into the ESP32 firmware

Your V3 code has something similar to:

const char* WIFI_SSID = "Navod_RDT";

const char* WIFI_PASSWORD = "your-password";

const char* RDT_API =

"http://192.168.137.25:5001/predict-rdt";

Upload the firmware.

Once you have established the hotspot configuration, try not to change it before the presentation.

One warning: the laptop's IP assigned by your phone can potentially change after reconnecting. Therefore, the evening before the viva, turn everything off/on and verify the IP again. If it changes, change RDT_API and re-upload the ESP32 firmware.

4. You need these files on the laptop

Keep a folder such as:

RDT_Server/

│

├── app.py

└── rdt_gradient_boosting_pp1.pkl

These two are what the live system actually needs.

The .pkl is your trained Gradient Boosting model.

app.py loads that model and exposes:

POST /predict-rdt

5. Start the Python server

Open PowerShell inside RDT_Server:

cd "D:\Shared Folder Laptop\SLIIT\4th Year 1st Sem\IT4010 - Research Project\RDT model\RDT_Server"

Then:

python app.py

Leave that terminal open.

You should see something like:

Running on http://127.0.0.1:5001

Running on http://192.168.x.x:5001

Yes — if you close this Python program, live RDT prediction stops.

But your chamber firmware should continue its local functions. Only ML/RDT becomes unavailable.

6. Verify the server before starting the chamber

On your laptop browser, open:

http://YOUR_LAPTOP_IP:5001/

You should see:

{

"message": "Cinnamon RDT Prediction API",

"status": "online"

}

Then you know:

hotspot → laptop → Flask → model = ready.

7. Power/start the ESP32

Now the ESP32 connects to the same phone hotspot:

Phone Hotspot

│

├──── Laptop

│

└──── ESP32

The USB cable between ESP32 and laptop is not carrying the ML requests. Wi-Fi is.

During drying, V3 sends the current drying state to the laptop, Python predicts RDT, and the result returns to the ESP32.

Press D:

TIME / RDT

Elapsed: 00:06

RDT: 2h 14m

ML: ONLINE