# Smart Cinnamon Dehydration Chamber Annotated Arduino Code

This document reproduces the supplied ESP32 S3 sketch with explanatory comments. The executable statements and pin assignments are unchanged. Read the notes below before using the relay outputs with a heater or fan.

# How the program works

At startup, it loads scale calibration, tares the empty tray, asks for initial and target moisture percentages, then captures the cinnamon’s starting mass. During drying it reads chamber temperature and humidity and estimates moisture from changing weight. At target, it turns the heater off, runs the fan for two minutes, and records CSV data on Serial.

# Important operating limits

The initial moisture percentage is entered by the user. The calculated moisture is an estimate based on that entry and the assumption that dry matter does not change; the SHT30 measures air relative humidity, not cinnamon moisture.

The present sketch does not read the earlier soil probe. Its RDT page explicitly says Pending.

The five target confirmations occur at approximately one-second sensor updates; a changing or mechanically unstable scale can still stop drying incorrectly. The code keeps the last weight if the HX711 stops responding while drying.

The 50 C cutoff and keypad 0 stop are software commands. On restart, wiring fault, or relay contact failure, this sketch alone cannot guarantee heater power is removed. Use an independent thermal cutoff and appropriately enclosed mains hardware.

The pin map in this file uses SHT SDA 6 SCL 5 and HX711 DT 16 SCK 15. Check it against the current physical wiring before uploading; older test sketches used different pins.

# Annotated sketch

Green comments are added explanations. Original comments and all executable statements follow the uploaded text. Cosmetic blank lines are condensed for readability.

// Wire supports the two I2C buses used for the SHT30 and OLED.

#include <Wire.h>

// GFX supplies drawing and text methods shared by Adafruit displays.

#include <Adafruit_GFX.h>

// SH110X drives the 128 x 64 SH1106G OLED in this sketch.

#include <Adafruit_SH110X.h>

// SHT31 library also reads the SHT30 family temperature and humidity sensor.

#include <Adafruit_SHT31.h>

// HX711 converts the load-cell signal into a weight reading.

#include <HX711.h>

// Keypad scans the four row and four column connections.

#include <Keypad.h>

// Preferences keeps calibration and batch count across ESP32 restarts.

#include <Preferences.h>

// =============================================================

// SMART CINNAMON DEHYDRATION CHAMBER

// ESP32-S3 DevKitC-1 N16R8

//

// FINAL PP1 DRYING FIRMWARE - V1

//

// CURRENT GPIO MAP

//

// SHT30:

//   SDA -> GPIO 6

//   SCL -> GPIO 5

//

// OLED:

//   SDA -> GPIO 8

//   SCL -> GPIO 9

//

// HX711:

//   DT  -> GPIO 16

//   SCK -> GPIO 15

//

// RELAYS:

//   Heater -> GPIO 10

//   Fan    -> GPIO 11

//

// KEYPAD ROWS:

//   38, 39, 40, 41

//

// KEYPAD COLS:

//   42, 47, 2, 1

//

// Relay board = ACTIVE LOW

// =============================================================

// =============================================================

// PIN DEFINITIONS

// =============================================================

// SHT30

// The SHT sensor is on its own I2C bus, separate from the OLED.

#define SHT_SDA 6

#define SHT_SCL 5

// OLED

// OLED I2C data and clock are GPIO 8 and 9.

#define OLED_SDA 8

#define OLED_SCL 9

// HX711

// HX711 data output and clock input connect to GPIO 16 and 15.

#define HX711_DT  16

#define HX711_SCK 15

// Relays

// GPIO 10 controls the heater relay; GPIO 11 controls the fan relay.

#define HEATER_RELAY_PIN 10

#define FAN_RELAY_PIN    11

// Active-low board: LOW energizes a relay and HIGH releases it.

#define RELAY_ON  LOW // active-low coil drive

#define RELAY_OFF HIGH // relay released

// =============================================================

// SETTINGS

// =============================================================

// Calibration reference

// Calibration assumes the reference mass really is 100 g.

const float CALIBRATION_WEIGHT_G = 100.0;

// Cooling / purge time after target MC reached

const unsigned long COOLING_TIME_MS = 120000UL; // 2 minutes // fan purge

// Sensor update

const unsigned long SENSOR_INTERVAL_MS = 1000UL; // about one sample per second

// OLED refresh

const unsigned long DISPLAY_INTERVAL_MS = 500UL; // refresh twice per second

// Research-data logging interval

const unsigned long LOG_INTERVAL_MS = 60000UL; // 1 minute // CSV sampling

// -------------------------------------------------------------

// Temperature protection

// -------------------------------------------------------------

// Heater turns OFF at/above this temperature.

// Heater may turn back ON after chamber falls below resume temp.

//

// This is SOFTWARE protection only.

// A physical thermal fuse / thermostat is still recommended.

// -------------------------------------------------------------

// Heat control has hysteresis: OFF at 50 C, eligible to restart at 47 C.

const float MAX_CHAMBER_TEMP_C = 50.0;

const float HEATER_RESUME_TEMP_C = 47.0; // thermostat lower threshold

// Require several consecutive target readings before stopping.

// Helps prevent one noisy load-cell reading from ending the batch.

// Five qualifying sensor cycles are required before cooling begins.

const int TARGET_CONFIRM_COUNT = 5;

// =============================================================

// I2C BUSES

// =============================================================

// SHT30 on GPIO 6 / 5

// Bus 0 is assigned to the SHT30.

TwoWire SHTBus = TwoWire(0);

// OLED on GPIO 8 / 9

// Bus 1 is assigned to the OLED.

TwoWire OLEDBus = TwoWire(1);

// =============================================================

// DEVICES

// =============================================================

// Create the OLED object with its width, height and dedicated I2C bus.

Adafruit_SH1106G display(

128,

64,

&OLEDBus,

-1

);

// Read the SHT sensor through SHTBus rather than the default Wire bus.

Adafruit_SHT31 sht31 =

Adafruit_SHT31(&SHTBus);

HX711 scale;

Preferences preferences;

// =============================================================

// KEYPAD

// =============================================================

const byte ROWS = 4;

const byte COLS = 4;

// Matrix order: 1 2 3 A; 4 5 6 B; 7 8 9 C; * 0 # D.

char keys[ROWS][COLS] = {

{'1', '2', '3', 'A'},

{'4', '5', '6', 'B'},

{'7', '8', '9', 'C'},

{'*', '0', '#', 'D'}

};

byte rowPins[ROWS] = {

38, 39, 40, 41

};

byte colPins[COLS] = {

42, 47, 2, 1

};

Keypad keypad = Keypad(

makeKeymap(keys),

rowPins,

colPins,

ROWS,

COLS

);

// =============================================================

// SYSTEM STATES

// =============================================================

// Only one workflow state is active at any moment.

enum SystemState {

STARTUP_TARE,

CALIBRATE_EMPTY,

CALIBRATE_WEIGHT,

ENTER_INITIAL_MC,

ENTER_TARGET_MC,

READY_TO_START,

DRYING,

COOLING,

COMPLETE,

ABORTED

};

SystemState systemState = STARTUP_TARE;

// =============================================================

// DISPLAY PAGE

// =============================================================

// Pages A to D are selectable while drying.

char activePage = 'A';

// A = Home

// B = Sensors

// C = Control status

// D = Time / RDT

// =============================================================

// SYSTEM VARIABLES

// =============================================================

bool oledOK = false;

bool shtOK = false;

bool hxOK = false;

bool heaterOn = false;

bool fanOn = false;

bool calibrationValid = false;

// Raw HX711 counts per gram after the 100 g calibration.

float calibrationFactor = 1.0;

// -------------------------------------------------------------

// Drying values

// -------------------------------------------------------------

float initialMoisture = 0.0;

float targetMoisture = 0.0;

float initialWeight = 0.0;

float currentWeight = 0.0;

// Estimated non-water mass, assumed constant through drying.

float dryMass = 0.0;

float targetWeight = 0.0;

// NAN signals that moisture cannot yet be calculated.

float estimatedMoisture = NAN;

// -------------------------------------------------------------

// Environmental values

// -------------------------------------------------------------

float chamberTemperature = NAN;

float chamberHumidity = NAN;

// Timestamp for detecting an unresponsive temperature sensor.

unsigned long lastValidSHTReading = 0;

// -------------------------------------------------------------

// Timing

// -------------------------------------------------------------

unsigned long dryingStartMs = 0;

unsigned long dryingDurationMs = 0;

unsigned long coolingStartMs = 0; // millis() at entry to COOLING

unsigned long lastSensorUpdate = 0;

unsigned long lastDisplayUpdate = 0;

unsigned long lastLogTime = 0;

// -------------------------------------------------------------

// Target confirmation

// -------------------------------------------------------------

// Counts consecutive readings at or below the moisture target.

int targetCounter = 0;

// -------------------------------------------------------------

// Input

// -------------------------------------------------------------

// Keypad digits accumulate here until * confirms a value.

String inputBuffer = "";

// -------------------------------------------------------------

// Batch number

// -------------------------------------------------------------

// Batch number is restored from nonvolatile preferences.

unsigned int batchCounter = 0;

String batchId = "B000";

// =============================================================

// OLED HELPER

// =============================================================

// Prepare a blank display buffer; each screen later calls display.display().

void clearOLED() {

// Avoid drawing if OLED initialization failed.

if (!oledOK)

return;

display.clearDisplay();

display.setTextColor(SH110X_WHITE);

display.setTextSize(1);

}

// =============================================================

// RELAY CONTROL

// =============================================================

// Update the state variable and physical heater output together.

void setHeater(bool on) {

heaterOn = on;

digitalWrite(

HEATER_RELAY_PIN,

// Choose the active-low level that matches the requested state.

on ? RELAY_ON : RELAY_OFF

);

}

// Update the state variable and physical fan output together.

void setFan(bool on) {

fanOn = on;

digitalWrite(

FAN_RELAY_PIN,

// Choose the active-low level that matches the requested state.

on ? RELAY_ON : RELAY_OFF

);

}

// Switch both outputs off for aborts and batch resets.

void stopEverything() {

setHeater(false);

setFan(false);

}

// =============================================================

// TIME FORMAT

// =============================================================

// Show elapsed milliseconds as zero-padded hours and minutes.

String formatTime(unsigned long milliseconds) {

unsigned long totalSeconds =

// Convert milliseconds to seconds before splitting hours and minutes.

milliseconds / 1000UL;

unsigned long hours =

totalSeconds / 3600UL;

unsigned long minutes =

(totalSeconds % 3600UL) / 60UL;

char buffer[12];

snprintf(

buffer,

sizeof(buffer),

"%02lu:%02lu",

hours,

minutes

);

return String(buffer);

}

// =============================================================

// CURRENT DRYING TIME

// =============================================================

// Freeze the drying time once cooling or an ending state starts.

unsigned long getDryingElapsed() {

if (systemState == DRYING) {

return millis() - dryingStartMs;

}

if (

systemState == COOLING ||

systemState == COMPLETE ||

systemState == ABORTED

) {

return dryingDurationMs;

}

return 0;

}

// =============================================================

// MOISTURE ESTIMATION

// =============================================================

//

// Wet-basis mass balance:

//

// DryMass = InitialWeight * (1 - InitialMC/100)

//

// CurrentMC =

// ((CurrentWeight - DryMass) / CurrentWeight) * 100

//

// =============================================================

// Estimate wet-basis moisture from current total weight and assumed dry mass.

float calculateEstimatedMoisture() {

if (

initialWeight <= 0 ||

currentWeight <= 0 ||

dryMass <= 0

) {

return NAN;

}

float mc =

((currentWeight - dryMass) /

currentWeight) * 100.0;

// Clamp impossible results caused by noise or a changing dry-mass assumption.

if (mc < 0)

mc = 0;

if (mc > 100)

mc = 100;

return mc;

}

// =============================================================

// SENSOR UPDATE

// =============================================================

// Called roughly once a second to refresh temperature, RH, weight and MC.

void updateSensors() {

// -----------------------------------------------------------

// SHT30

// -----------------------------------------------------------

// Ask the SHT30 for a fresh chamber temperature.

float temp =

sht31.readTemperature();

// Ask the SHT30 for relative humidity.

float hum =

sht31.readHumidity();

if (

!isnan(temp) &&

!isnan(hum)

) {

chamberTemperature = temp;

chamberHumidity = hum;

// Only successful paired SHT readings reset the 10-second watchdog.

lastValidSHTReading = millis();

}

// -----------------------------------------------------------

// LOAD CELL

// -----------------------------------------------------------

// Keep the previous weight when the HX711 is not ready.

if (scale.is_ready()) {

// Average several readings

float weight =

// Average five converted weight samples using the saved scale factor.

scale.get_units(5);

// Remove tiny zero drift

if (

weight > -0.5 &&

weight < 0.5

) {

weight = 0;

}

// The latest measured mass drives the moisture estimate.

currentWeight = weight;

}

// -----------------------------------------------------------

// ESTIMATED MOISTURE

// -----------------------------------------------------------

if (

systemState == DRYING ||

systemState == COOLING ||

systemState == COMPLETE

) {

// This is a weight-derived estimate, not a direct moisture measurement.

estimatedMoisture =

calculateEstimatedMoisture();

}

}

// =============================================================

// SHT SENSOR HEALTH

// =============================================================

// A reading older than 10 seconds counts as a failed temperature sensor.

bool shtSensorHealthy() {

if (!shtOK)

return false;

if (

millis() - lastValidSHTReading

> 10000UL

) {

return false;

}

return true;

}

// =============================================================

// DRYING CONTROL

// =============================================================

// Run the heater thermostat and check the weight-based stopping target.

void controlDrying() {

if (systemState != DRYING)

return;

// Fan remains ON during normal drying

setFan(true);

// -----------------------------------------------------------

// TEMPERATURE / SENSOR SAFETY

// -----------------------------------------------------------

// On missing temperature data, stop heat while continuing airflow.

if (!shtSensorHealthy()) {

// If chamber temperature sensor fails:

// heater OFF, fan ON.

setHeater(false);

}

else {

if (

chamberTemperature >=

MAX_CHAMBER_TEMP_C

) {

setHeater(false);

}

else if (

chamberTemperature <=

HEATER_RESUME_TEMP_C

) {

setHeater(true);

}

}

// -----------------------------------------------------------

// TARGET MOISTURE CHECK

// -----------------------------------------------------------

if (

!isnan(estimatedMoisture) &&

estimatedMoisture <= targetMoisture

) {

// Consecutive qualifying cycles help reject a single noisy value.

targetCounter++;

}

else {

// Any out-of-target reading breaks the confirmation streak.

targetCounter = 0;

}

// -----------------------------------------------------------

// TARGET CONFIRMED

// -----------------------------------------------------------

if (

targetCounter >=

TARGET_CONFIRM_COUNT

) {

dryingDurationMs =

millis() - dryingStartMs;

// Heater stops immediately

setHeater(false);

// Fan continues for 2 minutes

setFan(true);

// Mark the start of the timed cooling period.

coolingStartMs = millis();

systemState = COOLING;

Serial.println();

Serial.println(

"*** TARGET MOISTURE REACHED ***"

);

Serial.print(

"Final estimated MC: "

);

Serial.print(

estimatedMoisture,

2

);

Serial.println("%");

Serial.println(

"Heater OFF"

);

Serial.println(

"Fan cooling period started."

);

}

}

// =============================================================

// COOLING CONTROL

// =============================================================

// Keep fan on and heater off for a fixed two-minute purge.

void controlCooling() {

if (systemState != COOLING)

return;

setHeater(false);

setFan(true);

if (

millis() - coolingStartMs

>= COOLING_TIME_MS

) {

setFan(false);

systemState = COMPLETE;

Serial.println();

Serial.println(

"*** DRYING COMPLETE ***"

);

}

}

// =============================================================

// CSV RESEARCH DATA LOG

// =============================================================

// Describe the comma-separated records printed to Serial.

void printCSVHeader() {

Serial.println();

Serial.println(

"CSV_HEADER,"

"batchId,"

"elapsedSec,"

"temperatureC,"

"humidityPct,"

"weightG,"

"estimatedMC,"

"targetMC,"

"heater,"

"fan,"

"state"

);

}

// Convert the active state to a readable CSV label.

const char* stateName() {

if (systemState == DRYING)

return "DRYING";

if (systemState == COOLING)

return "COOLING";

if (systemState == COMPLETE)

return "COMPLETE";

if (systemState == ABORTED)

return "ABORTED";

return "SETUP";

}

// Log one comma-separated sample each minute while active.

void logResearchData() {

if (

systemState != DRYING &&

systemState != COOLING

) {

return;

}

// During cooling the logged drying duration stays frozen.

unsigned long elapsedSeconds =

getDryingElapsed() / 1000UL;

Serial.print("CSV,");

Serial.print(batchId);

Serial.print(",");

Serial.print(elapsedSeconds);

Serial.print(",");

Serial.print(chamberTemperature, 2);

Serial.print(",");

Serial.print(chamberHumidity, 2);

Serial.print(",");

Serial.print(currentWeight, 2);

Serial.print(",");

Serial.print(estimatedMoisture, 2);

Serial.print(",");

Serial.print(targetMoisture, 2);

Serial.print(",");

Serial.print(

heaterOn ? 1 : 0

);

Serial.print(",");

Serial.print(

fanOn ? 1 : 0

);

Serial.print(",");

Serial.println(

stateName()

);

}

// =============================================================

// STARTUP SCREEN

// =============================================================

// Startup requires either a saved calibration or a new 100 g calibration.

void showStartupScreen() {

clearOLED();

display.setCursor(0, 0);

display.println("LOAD CELL SETUP");

display.setCursor(0, 18);

display.println("Empty the tray");

display.setCursor(0, 34);

if (calibrationValid) {

display.println("* = Tare / Continue");

}

else {

display.println("Calibration needed");

}

display.setCursor(0, 50);

display.println("D = Calibrate 100g");

display.display();

}

// =============================================================

// CALIBRATION SCREENS

// =============================================================

// Ask the operator to remove all load before zeroing the tray.

void showCalibrationEmpty() {

clearOLED();

display.setCursor(0, 0);

display.println("CALIBRATION");

display.setCursor(0, 20);

display.println("EMPTY the tray");

display.setCursor(0, 38);

display.println("Press * to tare");

display.setCursor(0, 54);

display.println("# = Cancel");

display.display();

}

// Ask for the known 100 g reference at the tray center.

void showCalibrationWeight() {

clearOLED();

display.setCursor(0, 0);

display.println("CALIBRATION");

display.setCursor(0, 18);

display.println("Place 100g soap");

display.setCursor(0, 36);

display.println("in tray center");

display.setCursor(0, 52);

display.println("Press *");

display.display();

}

// =============================================================

// INPUT SCREENS

// =============================================================

// The initial MC is typed by the operator; it is not measured by the SHT30.

void showInitialMoistureScreen() {

clearOLED();

display.setCursor(0, 0);

display.println("Place cinnamon");

display.setCursor(0, 18);

display.println("Initial Moisture:");

display.setCursor(0, 38);

display.setTextSize(2);

if (inputBuffer.length() == 0) {

display.print("--");

}

else {

display.print(inputBuffer);

}

display.print("%");

display.setTextSize(1);

display.display();

}

// Collect the desired final moisture percentage.

void showTargetMoistureScreen() {

clearOLED();

display.setCursor(0, 0);

display.println("Target Moisture:");

display.setCursor(0, 20);

display.setTextSize(2);

if (inputBuffer.length() == 0) {

display.print("--");

}

else {

display.print(inputBuffer);

}

display.print("%");

display.setTextSize(1);

display.setCursor(0, 52);

display.println("* Confirm  # Back");

display.display();

}

// =============================================================

// READY SCREEN

// =============================================================

// Review input percentages and current weight before starting.

void showReadyScreen() {

clearOLED();

display.setCursor(0, 0);

display.print("Init:");

display.print(initialMoisture, 0);

display.print("%  Tgt:");

display.print(targetMoisture, 0);

display.println("%");

display.setCursor(0, 20);

display.print("Weight: ");

display.print(currentWeight, 1);

display.println("g");

display.setCursor(0, 39);

display.println("* = START DRYING");

display.setCursor(0, 54);

display.println("# = Back");

display.display();

}

// =============================================================

// PAGE A - HOME

// =============================================================

// Page A summarizes moisture, target, chamber conditions, mass and time.

void showHomeScreen() {

clearOLED();

display.setCursor(0, 0);

display.print("MC:");

if (isnan(estimatedMoisture))

display.print("--");

else

display.print(estimatedMoisture, 1);

display.print("% Tgt:");

display.print(targetMoisture, 0);

display.print("%");

display.setCursor(0, 16);

display.print("T:");

if (isnan(chamberTemperature))

display.print("--");

else

display.print(chamberTemperature, 1);

display.print("C RH:");

if (isnan(chamberHumidity))

display.print("--");

else

display.print(chamberHumidity, 0);

display.print("%");

display.setCursor(0, 32);

display.print("Wt:");

display.print(currentWeight, 1);

display.print("g");

display.setCursor(0, 48);

display.print("Time:");

display.print(

formatTime(

getDryingElapsed()

)

);

display.display();

}

// =============================================================

// PAGE B - SENSOR DETAILS

// =============================================================

// Page B displays raw chamber temperature, RH and scale weight.

void showSensorScreen() {

clearOLED();

display.setCursor(0, 0);

display.println("SENSOR DETAILS");

display.setCursor(0, 16);

display.print("Temp: ");

display.print(chamberTemperature, 1);

display.println(" C");

display.setCursor(0, 30);

display.print("RH:   ");

display.print(chamberHumidity, 1);

display.println(" %");

display.setCursor(0, 44);

display.print("Weight: ");

display.print(currentWeight, 1);

display.println("g");

display.setCursor(0, 56);

display.println("Ambient SHT: later");

display.display();

}

// =============================================================

// PAGE C - CONTROL STATUS

// =============================================================

// Page C shows relay states and the current state-machine mode.

void showControlScreen() {

clearOLED();

display.setCursor(0, 0);

display.println("CONTROL STATUS");

display.setCursor(0, 17);

display.print("Heater: ");

display.println(

heaterOn ? "ON" : "OFF"

);

display.setCursor(0, 31);

display.print("Fan:    ");

display.println(

fanOn ? "ON" : "OFF"

);

display.setCursor(0, 45);

display.print("Mode: ");

display.println(

stateName()

);

display.setCursor(0, 57);

display.println("0 = EMERGENCY STOP");

display.display();

}

// =============================================================

// PAGE D - TIME / RDT

// =============================================================

// Page D shows elapsed time and calculated target weight; RDT is pending.

void showTimeRDTScreen() {

clearOLED();

display.setCursor(0, 0);

display.println("TIME / RDT");

display.setCursor(0, 18);

display.print("Elapsed: ");

display.println(

formatTime(

getDryingElapsed()

)

);

display.setCursor(0, 34);

display.println("RDT: Pending");

display.setCursor(0, 50);

display.print("TargetWt:");

display.print(targetWeight, 1);

display.print("g");

display.display();

}

// =============================================================

// COOLING SCREEN

// =============================================================

// Display estimated MC and remaining fan purge seconds.

void showCoolingScreen() {

clearOLED();

display.setCursor(0, 0);

display.println("TARGET REACHED");

display.setCursor(0, 16);

display.println("Heater: OFF");

display.setCursor(0, 29);

display.println("Fan: COOLING");

unsigned long elapsed =

millis() - coolingStartMs;

unsigned long remaining = 0;

if (elapsed < COOLING_TIME_MS) {

remaining =

COOLING_TIME_MS - elapsed;

}

display.setCursor(0, 44);

display.print("MC: ");

display.print(

estimatedMoisture,

1

);

display.print("%");

display.setCursor(0, 56);

display.print("Remain: ");

display.print(

remaining / 1000UL

);

display.print("s");

display.display();

}

// =============================================================

// COMPLETE SCREEN

// =============================================================

// Display the final estimate and frozen drying duration.

void showCompleteScreen() {

clearOLED();

display.setCursor(0, 0);

display.println("DRYING COMPLETE");

display.setCursor(0, 18);

display.print("Final MC:");

display.print(

estimatedMoisture,

1

);

display.println("%");

display.setCursor(0, 34);

display.print("Time:");

display.println(

formatTime(

dryingDurationMs

)

);

display.setCursor(0, 50);

display.println("* = New Batch");

display.display();

}

// =============================================================

// ABORT SCREEN

// =============================================================

// Display the stopped state after the 0 key is pressed.

void showAbortedScreen() {

clearOLED();

display.setCursor(0, 8);

display.println("SESSION STOPPED");

display.setCursor(0, 28);

display.println("Heater OFF");

display.println("Fan OFF");

display.setCursor(0, 50);

display.println("* = New Batch");

display.display();

}

// =============================================================

// DISPLAY MANAGER

// =============================================================

// Choose the OLED view from the workflow state and active page.

void updateDisplayScreen() {

// Avoid drawing if OLED initialization failed.

if (!oledOK)

return;

switch (systemState) {

case STARTUP_TARE:

showStartupScreen();

break;

case CALIBRATE_EMPTY:

showCalibrationEmpty();

break;

case CALIBRATE_WEIGHT:

showCalibrationWeight();

break;

case ENTER_INITIAL_MC:

showInitialMoistureScreen();

break;

case ENTER_TARGET_MC:

showTargetMoistureScreen();

break;

case READY_TO_START:

showReadyScreen();

break;

case DRYING:

if (activePage == 'A')

showHomeScreen();

else if (activePage == 'B')

showSensorScreen();

else if (activePage == 'C')

showControlScreen();

else if (activePage == 'D')

showTimeRDTScreen();

break;

case COOLING:

showCoolingScreen();

break;

case COMPLETE:

showCompleteScreen();

break;

case ABORTED:

showAbortedScreen();

break;

}

}

// =============================================================

// START NEW BATCH

// =============================================================

// Clear batch measurements and return to the tray-tare stage.

void resetForNewBatch() {

// Request both relay outputs OFF in software.

stopEverything();

initialMoisture = 0;

targetMoisture = 0;

initialWeight = 0;

currentWeight = 0;

dryMass = 0;

targetWeight = 0;

estimatedMoisture = NAN;

dryingStartMs = 0;

dryingDurationMs = 0;

coolingStartMs = 0;

// Any out-of-target reading breaks the confirmation streak.

targetCounter = 0;

inputBuffer = "";

activePage = 'A';

systemState =

STARTUP_TARE;

}

// =============================================================

// START DRYING SESSION

// =============================================================

// Verify key sensors, capture starting weight, and derive target mass.

void startDryingSession() {

if (!scale.is_ready()) {

Serial.println(

"Cannot start: HX711 unavailable."

);

return;

}

// On missing temperature data, stop heat while continuing airflow.

if (!shtSensorHealthy()) {

Serial.println(

"Cannot start: SHT30 unavailable."

);

return;

}

// Capture accurate initial weight

currentWeight =

scale.get_units(20);

initialWeight =

currentWeight;

if (initialWeight < 5.0) {

Serial.println(

"ERROR: Initial weight too low."

);

return;

}

// -----------------------------------------------------------

// Calculate dry matter

// -----------------------------------------------------------

// Assumes initial wet-basis moisture supplied on keypad is accurate.

dryMass =

initialWeight *

(1.0 -

initialMoisture / 100.0);

// -----------------------------------------------------------

// Calculate predicted target weight

// -----------------------------------------------------------

// Solve wet-basis mass balance for the chosen target moisture.

targetWeight =

dryMass /

(1.0 -

targetMoisture / 100.0);

// This is a weight-derived estimate, not a direct moisture measurement.

estimatedMoisture =

initialMoisture;

// -----------------------------------------------------------

// Generate batch ID

// -----------------------------------------------------------

batchCounter++;

// Persist the new batch number after incrementing it.

preferences.putUInt(

"batchCount",

batchCounter

);

char batchBuffer[10];

snprintf(

batchBuffer,

sizeof(batchBuffer),

"B%03u",

batchCounter

);

batchId =

String(batchBuffer);

// -----------------------------------------------------------

// Start timer

// -----------------------------------------------------------

dryingStartMs =

millis();

// Any out-of-target reading breaks the confirmation streak.

targetCounter = 0;

activePage = 'A';

systemState =

DRYING;

// -----------------------------------------------------------

// Start drying

// -----------------------------------------------------------

setFan(true);

// Heater starts unless temperature already unsafe

if (

chamberTemperature <

MAX_CHAMBER_TEMP_C

) {

setHeater(true);

}

else {

setHeater(false);

}

// -----------------------------------------------------------

// Serial research information

// -----------------------------------------------------------

Serial.println();

Serial.println(

"================================"

);

Serial.println(

"DRYING SESSION STARTED"

);

Serial.print(

"Batch ID: "

);

Serial.println(batchId);

Serial.print(

"Initial moisture: "

);

Serial.print(

initialMoisture

);

Serial.println("%");

Serial.print(

"Target moisture: "

);

Serial.print(

targetMoisture

);

Serial.println("%");

Serial.print(

"Initial weight: "

);

Serial.print(

initialWeight,

2

);

Serial.println(" g");

Serial.print(

"Calculated dry mass: "

);

Serial.print(

dryMass,

2

);

Serial.println(" g");

Serial.print(

"Calculated target weight: "

);

Serial.print(

targetWeight,

2

);

Serial.println(" g");

Serial.println(

"================================"

);

printCSVHeader();

// Print one CSV row per minute, including during cooling.

lastLogTime =

millis() - LOG_INTERVAL_MS;

}

// =============================================================

// KEYPAD HANDLER

// =============================================================

// Dispatch a key according to the current state; keys have context-specific roles.

void handleKey(char key) {

Serial.print(

"KEY: "

);

Serial.println(key);

// ===========================================================

// STARTUP / TARE

// ===========================================================

if (

systemState ==

STARTUP_TARE

) {

if (key == 'D') {

systemState =

CALIBRATE_EMPTY;

return;

}

if (key == '*') {

if (!calibrationValid) {

Serial.println(

"Calibration required. Press D."

);

return;

}

if (!scale.is_ready()) {

Serial.println(

"HX711 not ready."

);

return;

}

Serial.println(

"Taring empty tray..."

);

scale.set_scale(

calibrationFactor

);

// Average 20 unloaded samples to establish the current zero point.

scale.tare(20);

Serial.println(

"Tare complete."

);

inputBuffer = "";

systemState =

ENTER_INITIAL_MC;

return;

}

}

// ===========================================================

// CALIBRATION - EMPTY TRAY

// ===========================================================

if (

systemState ==

CALIBRATE_EMPTY

) {

if (key == '#') {

systemState =

STARTUP_TARE;

return;

}

if (key == '*') {

// Temporarily expose unscaled HX711 counts during calibration.

scale.set_scale(1.0);

// Average 20 unloaded samples to establish the current zero point.

scale.tare(20);

systemState =

CALIBRATE_WEIGHT;

return;

}

}

// ===========================================================

// CALIBRATION - 100g

// ===========================================================

if (

systemState ==

CALIBRATE_WEIGHT

) {

if (key == '#') {

systemState =

STARTUP_TARE;

return;

}

if (key == '*') {

Serial.println(

"Calibrating with 100g..."

);

delay(1500);

// Temporarily expose unscaled HX711 counts during calibration.

scale.set_scale(1.0);

float raw =

scale.get_units(30);

// Divide the raw difference by 100 g to obtain counts per gram.

calibrationFactor =

raw /

CALIBRATION_WEIGHT_G;

scale.set_scale(

calibrationFactor

);

// Save the new factor to flash for the next boot.

preferences.putFloat(

"calFactor",

calibrationFactor

);

calibrationValid =

true;

// Read the reference again to show the post-calibration result.

float verification =

scale.get_units(20);

Serial.print(

"Calibration factor: "

);

Serial.println(

calibrationFactor,

6

);

Serial.print(

"Verification weight: "

);

Serial.print(

verification,

2

);

Serial.println(" g");

delay(1500);

systemState =

STARTUP_TARE;

return;

}

}

// ===========================================================

// INITIAL MOISTURE INPUT

// ===========================================================

if (

systemState ==

ENTER_INITIAL_MC

) {

if (

key >= '0' &&

key <= '9'

) {

if (

inputBuffer.length()

< 3

) {

inputBuffer += key;

}

return;

}

if (key == '#') {

inputBuffer = "";

return;

}

if (key == '*') {

if (

inputBuffer.length()

== 0

) {

return;

}

float value =

inputBuffer.toFloat();

if (

value <= 0 ||

value >= 100

) {

Serial.println(

"Invalid initial moisture."

);

inputBuffer = "";

return;

}

initialMoisture =

value;

inputBuffer = "";

systemState =

ENTER_TARGET_MC;

return;

}

}

// ===========================================================

// TARGET MOISTURE INPUT

// ===========================================================

if (

systemState ==

ENTER_TARGET_MC

) {

if (

key >= '0' &&

key <= '9'

) {

if (

inputBuffer.length()

< 3

) {

inputBuffer += key;

}

return;

}

if (key == '#') {

inputBuffer = "";

systemState =

ENTER_INITIAL_MC;

return;

}

if (key == '*') {

if (

inputBuffer.length()

== 0

) {

return;

}

float value =

inputBuffer.toFloat();

if (

value <= 0 ||

value >= initialMoisture

) {

Serial.println(

"Target must be below initial MC."

);

inputBuffer = "";

return;

}

targetMoisture =

value;

inputBuffer = "";

systemState =

READY_TO_START;

return;

}

}

// ===========================================================

// READY TO START

// ===========================================================

if (

systemState ==

READY_TO_START

) {

if (key == '#') {

inputBuffer = "";

systemState =

ENTER_TARGET_MC;

return;

}

if (key == '*') {

startDryingSession();

return;

}

}

// ===========================================================

// ACTIVE DRYING

// ===========================================================

if (

systemState == DRYING

) {

if (

key == 'A' ||

key == 'B' ||

key == 'C' ||

key == 'D'

) {

activePage = key;

return;

}

// Emergency stop

// Pressing 0 stops heater and fan immediately in active states.

if (key == '0') {

dryingDurationMs =

millis() -

dryingStartMs;

// Request both relay outputs OFF in software.

stopEverything();

systemState =

ABORTED;

Serial.println(

"*** EMERGENCY STOP ***"

);

return;

}

}

// ===========================================================

// COOLING

// ===========================================================

if (

systemState ==

COOLING

) {

// Pressing 0 stops heater and fan immediately in active states.

if (key == '0') {

// Request both relay outputs OFF in software.

stopEverything();

systemState =

ABORTED;

return;

}

}

// ===========================================================

// COMPLETE / ABORTED

// ===========================================================

if (

systemState == COMPLETE ||

systemState == ABORTED

) {

if (key == '*') {

resetForNewBatch();

return;

}

}

}

// =============================================================

// SETUP

// =============================================================

// Arduino runs setup once at boot to initialize outputs, buses and saved data.

void setup() {

// Open serial reporting at 115200 bits per second.

Serial.begin(115200);

delay(1200);

Serial.println();

Serial.println(

"================================"

);

Serial.println(

"SMART CINNAMON DEHYDRATION"

);

Serial.println(

"SYSTEM STARTING"

);

Serial.println(

"================================"

);

// ===========================================================

// RELAYS - SAFE OFF DURING BOOT

// ===========================================================

pinMode(

HEATER_RELAY_PIN,

OUTPUT

);

pinMode(

FAN_RELAY_PIN,

OUTPUT

);

// Request both relay outputs OFF in software.

stopEverything();

// ===========================================================

// OLED

// ===========================================================

// Start OLED bus on GPIO 8 and 9 at 100 kHz.

OLEDBus.begin(

OLED_SDA,

OLED_SCL,

100000

);

oledOK =

// Probe SH1106 display at address 0x3C.

display.begin(

0x3C,

true

);

Serial.print(

"OLED: "

);

Serial.println(

oledOK ?

"OK" :

"FAIL"

);

// ===========================================================

// SHT30

// ===========================================================

// Start SHT bus on GPIO 6 and 5 at 100 kHz.

SHTBus.begin(

SHT_SDA,

SHT_SCL,

100000

);

shtOK =

// Probe the SHT30 at its expected I2C address.

sht31.begin(0x44);

Serial.print(

"SHT30: "

);

Serial.println(

shtOK ?

"OK" :

"FAIL"

);

if (shtOK) {

lastValidSHTReading =

millis();

}

// ===========================================================

// HX711

// ===========================================================

// HX711 DOUT is GPIO 16; SCK is GPIO 15.

scale.begin(

HX711_DT,

HX711_SCK

);

delay(500);

hxOK =

scale.is_ready();

Serial.print(

"HX711: "

);

Serial.println(

hxOK ?

"OK" :

"FAIL"

);

// ===========================================================

// PREFERENCES

// ===========================================================

// Open the nonvolatile storage namespace named loadcell.

preferences.begin(

"loadcell",

false

);

// Divide the raw difference by 100 g to obtain counts per gram.

calibrationFactor =

// Restore prior calibration, or zero if none was stored.

preferences.getFloat(

"calFactor",

0.0

);

if (

fabs(calibrationFactor)

> 0.0001

) {

calibrationValid =

true;

scale.set_scale(

calibrationFactor

);

Serial.print(

"Saved calibration factor: "

);

Serial.println(

calibrationFactor,

6

);

}

else {

calibrationValid =

false;

Serial.println(

"No saved calibration."

);

}

batchCounter =

// Restore the persistent batch number, defaulting to zero.

preferences.getUInt(

"batchCount",

0

);

// ===========================================================

// INITIAL SENSOR READ

// ===========================================================

updateSensors();

// ===========================================================

// STARTUP OLED

// ===========================================================

if (oledOK) {

clearOLED();

display.setCursor(0, 8);

display.println(

"SMART CINNAMON"

);

display.setCursor(0, 25);

display.println(

"DEHYDRATION"

);

display.setCursor(0, 45);

display.println(

"Starting..."

);

display.display();

delay(1500);

}

systemState =

STARTUP_TARE;

updateDisplayScreen();

}

// =============================================================

// LOOP

// =============================================================

// Repeatedly scan keys and schedule sensing, display and logging with millis().

void loop() {

// -----------------------------------------------------------

// KEYPAD

// -----------------------------------------------------------

char key =

// A nonzero character means one key press was detected.

keypad.getKey();

if (key) {

handleKey(key);

updateDisplayScreen();

}

// -----------------------------------------------------------

// SENSOR UPDATE

// -----------------------------------------------------------

if (

millis() -

lastSensorUpdate

>= SENSOR_INTERVAL_MS

) {

// Remember this sampling instant for the next 1-second interval.

lastSensorUpdate =

millis();

updateSensors();

controlDrying();

controlCooling();

}

// -----------------------------------------------------------

// OLED UPDATE

// -----------------------------------------------------------

if (

millis() -

lastDisplayUpdate

>= DISPLAY_INTERVAL_MS

) {

// Refresh the OLED roughly twice per second.

lastDisplayUpdate =

millis();

updateDisplayScreen();

}

// -----------------------------------------------------------

// RESEARCH DATA LOGGING

// -----------------------------------------------------------

if (

(

systemState == DRYING ||

systemState == COOLING

) &&

(

millis() -

lastLogTime

>= LOG_INTERVAL_MS

)

) {

// Print one CSV row per minute, including during cooling.

lastLogTime =

millis();

logResearchData();

}

}